CERTIFICATE OF ANALYSIS

SECTION 1. PRODUCT AND COMPANY IDENTIFICATION

Product name:   Cosmetic Grade Glitter

Product code:    FCHL601

Production date:  Nov. 1, 2020 ~ Dec. 24, 2020

Expiry date:      Dec. 23, 2023

Supplier:        Zhongshan Zhongtai Pigment Co., Ltd.

SECTION 2. INFORMATION ON SPECIFICATION

| Product Code | Color | Thickness（μm） | Solvent- resistance | Heat Resistance | Light Fastness | Shape | Size（mm） | FD&C Colorant | D&C Colorant | Pigment | PH Value | Surface Degradability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FCHL601 | Greenish Yellow | 25/50 | yes | 160~180℃ | 6-7 | Hexagon | 0.2 | ✔ | / | / | 6.0~7.5 | no |

SECTION 3. INFORMATION ON INGREDIENTS

| Key Ingredient | CAS NO. | EC NO. | Target% |
| --- | --- | --- | --- |
| Polyethylene terephthalate | 25038-59-9 | 607-507-1 | 97.02 |
| Acrylates copolymer | 25035-69-2 | 607-492-1 | 2.6 |
| Aluminum | 7429-90-5 | 231-072-3 | 0.01 |
| FD&C Blue 1（C.I. 42090） | 3844-45-9 | 223-339-8 | 0.13 |
| FD&C Yellow 5 （CI.19140） | 1934-21-0 | 217-699-5 | 0.24 |

SECTION 4. TEST REPORT ON HEAVY METALS

The test results are as follows:

Remarks : (1). 1 mg/kg = 0.0001%    (2). MDL = Method Detection Limit     （3). ND = Not Detected ( < MDL )

German Health Authority BgA (Recommendation from German Health Journal No.28, July 1985)- Heavy Metals

Test Method : SGS in house method(SHTC® CHEM® SOP ®169®T), analysis was performed by ICP®MS.

| Test Item(s) | Unit | MDL | Test Result(s) | Limit |
| --- | --- | --- | --- | --- |
| Lead (Pb) | mg/kg | 0.2 | ND | 20 |
| Cadmium (Cd) | mg/kg | 0.2 | ND | 5 |
| Arsenic (As) | mg/kg | 0.2 | ND | 5 |
| mercury(Hg) | mg/kg | 0.2 | ND | 1 |
| Antimony (Sb) | mg/kg | 0.2 | ND | 10 |

German Health Journal No. 7/1992, Session 45 from November 14, 1991®Soluble Nickel

Test Method : Sample was extracted in 0.07mol/L HCI at 37℃,analysis was performed by ICP®OES.

| Test Item(s) | Unit | MDL | Test Result(s) | Limit |
| --- | --- | --- | --- | --- |
| Soluble Nickel (Ni) | mg/kg | 5 | ND | 10 |

To determine Lead content in the submitted sample in accordance with the CA Prop. 65.

Test Method : With reference to Inhouse method, analysis was performed by ICP-MS/ICP-OES.

| Test item(s) | Unit(s) | MDL | Test result(s) | Requirement(s) |
| --- | --- | --- | --- | --- |
| *Lead | mg/kg | 0.2 | ND | < 10 |

* With refer to California proposition 65 announced on April 22, 2009 (Case no. H217587 (Consolidate with No. 01- 032306)), lead content will have to contain less than 5mg/kg for lipsticks and lipliners, while 10 mg/kg for eyeshadows and blushes.

To determine Mercury content in the submitted sample in accordance with the US FDA 21 CFR 700.13.

Test Method : With reference to Inhouse method, analysis was performed by ICP-MS/ICP-OES.

| Test item(s) | Unit(s) | MDL | Test result(s) | Requirement(s) |
| --- | --- | --- | --- | --- |
| Mercury | mg/kg | 0.2 | ND | < 1 |

Chromium

Test Method : With reference to Inhouse method, analysis was performed by ICP-MS/ICP-OES.

| Test Item(s) | Unit | MDL | Test Result(s) |
| --- | --- | --- | --- |
| Chromium (Cr) | mg/kg | 0.2 | ND |